SOURCE CODE:

/*

* ============================================================

*  Smart Helmet Accident Alert System — Arduino IDE Version

* ============================================================

*

*  HARDWARE vs Wokwi:

*   - Push button (GPIO18) REPLACED with real SW420 vibration sensor

*   - FSR pressure sensor  REMOVED → fixed dummy value (500)

*   - NEO-6M GPS module    REMOVED → hardcoded Nagpur coordinates

*

*  DATA FLOW (no conflict):

*   ESP32 → HiveMQ MQTT → Node-RED → ThingsBoard

*   ESP32 never calls ThingsBoard directly.

*   Node-RED is the only one that POSTs to ThingsBoard.

*

*/

#include <WiFi.h>

#include <PubSubClient.h>

#include <Wire.h>          // I2C communication for MPU6050

#include <ArduinoJson.h>   // JSON serialization for MQTT payloads

#include <math.h>          // sqrt(), atan2(), degrees()

// ── WiFi Credentials ─────────────────────────────────────────

const char* WIFI_SSID     = "Viraj";

const char* WIFI_PASSWORD = "30072005";

// ── MQTT Broker Settings ──────────────────────────────────────

const char* MQTT_BROKER  = "broker.hivemq.com";   // Public HiveMQ broker

const int   MQTT_PORT    = 1883;                   // Standard MQTT port (unencrypted)

const char* MQTT_CLIENT  = "SmartHelmet_Arduino_01"; // Unique client ID

const char* TOPIC_TELE   = "smarthelmet/telemetry";  // Live sensor data topic

const char* TOPIC_ALERT  = "smarthelmet/accident";   // Accident alert topic

const char* TOPIC_STATUS = "smarthelmet/status";     // System status topic

// ── GPIO Pin Definitions ──────────────────────────────────────

#define SDA_PIN        21   // I2C data line for MPU6050

#define SCL_PIN        22   // I2C clock line for MPU6050

#define BTN_CANCEL     14   // Cancel button (INPUT_PULLUP — LOW when pressed)

#define SW420_PIN      13   // SW420 vibration sensor (HIGH = shock detected)

#define LED_GPS        32   // Green LED — indicates system/GPS ready

#define LED_ACCIDENT   12   // Red LED — blinks on accident detection

// ── MPU6050 I2C Address ───────────────────────────────────────

#define MPU_ADDR       0x68  // Default I2C address when AD0 pin is LOW

// ── Detection Thresholds ──────────────────────────────────────

#define ACCEL_THRESH   2.5f   // g-force above this triggers accident check

#define TILT_THRESH    60.0  // Tilt angle (degrees) above this triggers accident check

#define CANCEL_MS      10000  // 10-second window for rider to cancel alert

// ── Hardcoded GPS Location (Nagpur) ──────────────────────────

// No GPS module used — coordinates are fixed for demonstration

const float GPS_LAT = 21.145800;

const float GPS_LNG = 79.088200;

// ── Fixed Pressure Value (no FSR sensor) ─────────────────────

const int FIXED_PRESSURE = 500;

// ── Global Objects & State Variables ─────────────────────────

WiFiClient   wifiClient;       // Base TCP client for MQTT

PubSubClient mqtt(wifiClient); // MQTT client wrapping the WiFi connection

bool accidentActive = false;   // Prevents re-triggering during active accident

int  sampleCount    = 0;       // Total loop iterations (for serial logging)

int  frameCount     = 0;       // Frame counter included in telemetry payload

// ── Pre-Accident Rolling Buffer ───────────────────────────────

// Stores the last 10 sensor samples before an accident is confirmed

struct Sample {

float accel, tilt;

int   pressure, vibration;

unsigned long ts;  // Timestamp from millis()

};

Sample preBuffer[10]; // Circular buffer — always holds latest 10 frames

int preIdx = 0;       // Current write index in the circular buffer

// ════════════════════════════════════════════════════════════

//  FUNCTION DECLARATIONS

// ════════════════════════════════════════════════════════════

void connectWiFi();

void connectMQTT();

void initMPU6050();

void readMPU6050(float &accelMag, float &tiltAngle);

void handleAccident(float accel, float tilt, int pressure);

void publishStatus(const char* status, const char* msg);

// ════════════════════════════════════════════════════════════

//  SETUP — runs once on boot

// ════════════════════════════════════════════════════════════

void setup() {

Serial.begin(115200);  // Initialize serial monitor at 115200 baud

delay(500);

Wire.begin(SDA_PIN, SCL_PIN);  // Start I2C bus on GPIO 21/22

// Configure GPIO pin modes

pinMode(BTN_CANCEL,   INPUT_PULLUP);  // Internal pull-up; button press → LOW

pinMode(SW420_PIN,    INPUT);         // SW420 floats HIGH on vibration

pinMode(LED_GPS,      OUTPUT);

pinMode(LED_ACCIDENT, OUTPUT);

// Ensure all outputs start LOW (off)

digitalWrite(LED_ACCIDENT, LOW);

digitalWrite(LED_GPS,      LOW);

initMPU6050();   // Wake up and configure MPU6050 via I2C

connectWiFi();   // Connect to WiFi network

// Configure and connect MQTT broker

mqtt.setServer(MQTT_BROKER, MQTT_PORT);

mqtt.setBufferSize(512);  // Increase buffer to handle larger JSON payloads

connectMQTT();

// 4 blinks on green LED = system is ready

for (int i = 0; i < 4; i++) {

digitalWrite(LED_GPS, HIGH); delay(150);

digitalWrite(LED_GPS, LOW);  delay(150);

}

digitalWrite(LED_GPS, HIGH);  // Keep green LED ON = GPS/system ready

publishStatus("ONLINE", "Smart Helmet system started");

Serial.println("\n==============================================");

Serial.println("  Smart Helmet — Arduino IDE");

Serial.println("----------------------------------------------");

Serial.println("  GPS    : Nagpur hardcoded (21.1458, 79.0882)");

Serial.println("  FSR    : Fixed value 500 (sensor not used)");

Serial.println("  VIB    : SW420 on GPIO18 (HIGH = shock)");

Serial.println("----------------------------------------------");

Serial.println("  DATA FLOW:");

Serial.println("  ESP32 → HiveMQ → Node-RED → ThingsBoard");

Serial.println("----------------------------------------------");

Serial.println("  TRIGGER ACCIDENT:");

Serial.println("  Shake/tilt MPU6050 hard AND shock SW420");

Serial.println("  Press GPIO4 button within 10s to cancel");

Serial.println("==============================================\n");

}

// ════════════════════════════════════════════════════════════

//  LOOP — runs continuously every ~1 second

// ════════════════════════════════════════════════════════════

void loop() {

if (!mqtt.connected()) connectMQTT();  // Reconnect MQTT if connection dropped

mqtt.loop();  // Process incoming MQTT messages and keep connection alive

float accelMag, tiltAngle;

readMPU6050(accelMag, tiltAngle);  // Read raw data and compute g-force + tilt

int  pressure  = FIXED_PRESSURE;                    // Use fixed FSR value (no sensor)

bool vibration = (digitalRead(SW420_PIN) == HIGH);  // HIGH = shock detected by SW420

sampleCount++;

frameCount++;

// Store current frame into the circular pre-accident buffer

preBuffer[preIdx] = { accelMag, tiltAngle, pressure, vibration ? 1 : 0, millis() };

preIdx = (preIdx + 1) % 10;  // Advance index; wraps back to 0 after index 9

// Accident requires ALL THREE conditions to be true simultaneously

bool highAccel = (accelMag  > ACCEL_THRESH);  // Sudden impact force

bool highTilt  = (tiltAngle > TILT_THRESH);   // Helmet tilted beyond normal range

bool vibHit    = vibration;                   // Physical shock registered by SW420

bool accident  = highAccel && highTilt && vibHit;

// ── Build and publish telemetry JSON to MQTT ──────────────

StaticJsonDocument<300> doc;

doc["acceleration"] = round(accelMag  * 100) / 100.0;  // Round to 2 decimal places

doc["tilt_angle"]   = round(tiltAngle * 10)  / 10.0;   // Round to 1 decimal place

doc["vibration"]    = vibration ? 1 : 0;

doc["pressure"]     = pressure;

doc["latitude"]     = GPS_LAT;

doc["longitude"]    = GPS_LNG;

doc["accident"]     = accident;

doc["severity"]     = accident ? (accelMag > 4.0 ? "HIGH" : "MEDIUM") : "NORMAL";

doc["frame"]        = frameCount;

doc["device_id"]    = "SmartHelmet_Arduino";

char buf[300];

serializeJson(doc, buf);

mqtt.publish(TOPIC_TELE, buf, false);  // false = don't retain message on broker

// Serial log showing sensor values and which thresholds are triggered

Serial.printf("[t=%03d] accel=%.2fg | tilt=%.1fdeg | SW420=%d | fsr=%d(fixed)",

sampleCount, accelMag, tiltAngle, vibration ? 1 : 0, pressure);

Serial.printf("  [A:%s T:%s V:%s]",

highAccel ? "HIT" : "ok",

highTilt  ? "HIT" : "ok",

vibHit    ? "HIT" : "ok");

if (accident) Serial.print("  <<< ACCIDENT!");

Serial.println();

// Only trigger accident handler if not already in an active accident sequence

if (!accidentActive && accident) {

handleAccident(accelMag, tiltAngle, pressure);

}

delay(1000);  // 1-second sampling interval

}

// ════════════════════════════════════════════════════════════

//  ACCIDENT HANDLER

//  Called once when all 3 conditions trigger simultaneously.

//  Gives rider 10s to cancel; if not cancelled, fires MQTT alert.

// ════════════════════════════════════════════════════════════

void handleAccident(float accel, float tilt, int pressure) {

accidentActive = true;       // Lock out further triggers during this sequence

digitalWrite(LED_ACCIDENT, HIGH);  // Turn red LED on immediately

// Print accident details to serial

Serial.println("\n!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!");

Serial.println("!!        ACCIDENT DETECTED              !!");

Serial.println("!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!");

Serial.printf("  Acceleration : %.2f g\n",   accel);

Serial.printf("  Tilt Angle   : %.1f deg\n", tilt);

Serial.println("  SW420        : SHOCK DETECTED");

Serial.printf("  Pressure     : %d (fixed — no FSR)\n", pressure);

Serial.printf("  GPS          : %.6f, %.6f (Nagpur hardcoded)\n", GPS_LAT, GPS_LNG);

Serial.println();

// Dump the last 10 pre-accident frames from the circular buffer

Serial.println("[EVENT LOG] Pre-accident frames (last 10s):");

for (int i = 0; i < 10; i++) {

int idx = (preIdx + i) % 10;  // Read from oldest to newest

Serial.printf("  pre_%02d | accel=%.2fg | tilt=%.1fdeg\n",

i + 1, preBuffer[idx].accel, preBuffer[idx].tilt);

}

Serial.println();

Serial.println(" Press GPIO4 button within 10s to CANCEL");

// ── 10-second cancel window ──────────────────────────────

unsigned long start     = millis();

bool          cancelled = false;

while (millis() - start < CANCEL_MS) {

mqtt.loop();  // Keep MQTT alive during the wait loop

if (digitalRead(BTN_CANCEL) == LOW) {  // Button pressed = rider is conscious

cancelled = true;

Serial.println("[CANCEL] Alert cancelled by rider.");

publishStatus("CANCELLED", "Rider cancelled alert");

break;

}

// Blink red LED every 400ms during cancel window

digitalWrite(LED_ACCIDENT, (millis() / 400) % 2);

delay(50);

}

if (!cancelled) {

// ── Build alert strings ───────────────────────────────

String severity = (accel > 4.0) ? "HIGH" : "MEDIUM";  // Classify impact severity

String mapsUrl  = "https://maps.google.com/?q=" +

String(GPS_LAT, 6) + "," + String(GPS_LNG, 6);  // Google Maps deep link

Serial.println("[ALERT] Sending MQTT alert...");

// ── Build full accident alert JSON payload ────────────

StaticJsonDocument<400> alert;

alert["ACCIDENT"]     = true;

alert["acceleration"] = round(accel * 100) / 100.0;

alert["tilt_angle"]   = round(tilt  * 10)  / 10.0;

alert["vibration"]    = 1;

alert["pressure"]     = pressure;

alert["latitude"]     = GPS_LAT;

alert["longitude"]    = GPS_LNG;

alert["severity"]     = severity;

alert["device_id"]    = "SmartHelmet_Arduino";

alert["maps_link"]    = mapsUrl;

alert["message"]      = "ACCIDENT DETECTED - Emergency Alert";

char alertBuf[400];

serializeJson(alert, alertBuf);

// Publish 3 times to both topics for reliability (Node-RED picks this up)

for (int i = 0; i < 3; i++) {

mqtt.publish(TOPIC_ALERT, alertBuf, false);

mqtt.publish(TOPIC_TELE,  alertBuf, false);

delay(300);

}

Serial.println("[MQTT] Published to smarthelmet/accident");

serializeJsonPretty(alert, Serial);  // Pretty-print full payload to serial

Serial.println();

// ── Publish 10 post-accident decay frames ────────────

// Simulates sensor readings gradually returning to normal after impact

Serial.println("[EVENT LOG] Post-accident frames:");

for (int i = 1; i <= 10; i++) {

float decayAccel = max(0.5f, accel - (i * 0.38f));  // Accel decays toward 0.5g

float decayTilt  = max(10.0f, tilt  - (i * 7.0f)); // Tilt decays toward 10deg

Serial.printf("  post_%02d | accel=%.2fg | tilt=%.1fdeg\n",

i, decayAccel, decayTilt);

StaticJsonDocument<200> post;

post["acceleration"] = round(decayAccel * 100) / 100.0;

post["tilt_angle"]   = round(decayTilt  * 10)  / 10.0;

post["vibration"]    = 1;

post["pressure"]     = max(100, pressure - i * 40);  // Pressure decays toward 100

post["latitude"]     = GPS_LAT;

post["longitude"]    = GPS_LNG;

post["accident"]     = true;

post["phase"]        = "post";  // Tag frame as post-accident phase

post["frame"]        = i;

char postBuf[200];

serializeJson(post, postBuf);

mqtt.publish(TOPIC_TELE, postBuf, false);

mqtt.loop();   // Keep MQTT alive between frame publishes

delay(600);    // 600ms between post-accident frames

}

Serial.println("\n==========================================");

Serial.println("  EMERGENCY ALERT SENT SUCCESSFULLY");

Serial.println("  MQTT  → Node-RED → ThingsBoard");

Serial.println("  20 frames logged (10 pre + 10 post)");

Serial.println("==========================================");

}

// Reset indicators and unlock accident detection

digitalWrite(LED_ACCIDENT, LOW);

digitalWrite(LED_GPS,      HIGH);  // Restore green LED

accidentActive = false;            // Allow new accident to be detected

Serial.println("\n[SYSTEM] Resuming normal monitoring...\n");

}

// ════════════════════════════════════════════════════════════

//  READ MPU6050

//  Reads raw 16-bit accelerometer values over I2C from register 0x3B.

//  Converts to g-force magnitude and tilt angle in degrees.

// ════════════════════════════════════════════════════════════

void readMPU6050(float &accelMag, float &tiltAngle) {

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x3B);              // Point to ACCEL_XOUT_H register

Wire.endTransmission(false);   // Repeated start — keep bus active

Wire.requestFrom(MPU_ADDR, 6, true);  // Read 6 bytes: X, Y, Z (2 bytes each)

// Combine high and low bytes into signed 16-bit integers

int16_t ax = (Wire.read() << 8) | Wire.read();

int16_t ay = (Wire.read() << 8) | Wire.read();

int16_t az = (Wire.read() << 8) | Wire.read();

// Convert raw ADC values to g-force (±2g range → 16384 LSB/g)

float x = ax / 16384.0f;

float y = ay / 16384.0f;

float z = az / 16384.0f;

accelMag  = sqrt(x*x + y*y + z*z);              // Total acceleration magnitude in g

tiltAngle = degrees(atan2(sqrt(x*x + y*y), abs(z))); // Tilt angle from vertical axis

}

// ════════════════════════════════════════════════════════════

//  WiFi CONNECT

// ════════════════════════════════════════════════════════════

void connectWiFi() {

Serial.printf("[WiFi] Connecting to %s", WIFI_SSID);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

int tries = 0;

while (WiFi.status() != WL_CONNECTED && tries < 30) {

delay(500); Serial.print("."); tries++;  // Retry up to 30 times (~15 seconds)

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\n[WiFi] Connected! IP: " + WiFi.localIP().toString());

} else {

Serial.println("\n[WiFi] FAILED — check SSID and password");

}

}

// ════════════════════════════════════════════════════════════

//  MQTT CONNECT

// ════════════════════════════════════════════════════════════

void connectMQTT() {

Serial.print("[MQTT] Connecting to broker.hivemq.com");

int tries = 0;

while (!mqtt.connected() && tries < 5) {  // Attempt up to 5 reconnections

if (mqtt.connect(MQTT_CLIENT)) {

Serial.println(" Connected!");

mqtt.subscribe(TOPIC_STATUS);  // Subscribe to receive status commands

} else {

Serial.printf(" failed rc=%d, retrying...\n", mqtt.state());

delay(2000);  // Wait 2s before retrying

}

tries++;

}

}

// ════════════════════════════════════════════════════════════

//  HELPERS

// ════════════════════════════════════════════════════════════

// Publishes a simple status JSON to the smarthelmet/status topic

void publishStatus(const char* status, const char* msg) {

StaticJsonDocument<128> doc;

doc["status"]  = status;

doc["message"] = msg;

char buf[128];

serializeJson(doc, buf);

mqtt.publish(TOPIC_STATUS, buf);

}

// Wakes MPU6050 by writing 0x00 to the PWR_MGMT_1 register (0x6B)

// By default the MPU6050 boots in sleep mode — this clears the sleep bit

void initMPU6050() {

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B);   // PWR_MGMT_1 register

Wire.write(0x00);   // Clear sleep bit → sensor becomes active

Wire.endTransmission(true);

Serial.println("[INIT] MPU6050 ready");

}